1.

Create a comparison table listing the main differences between DDR3, DDR4, and DDR5 RAM, including speed, voltage, and typical use cases.

## DDR3 vs. DDR4 vs. DDR5

| Feature | DDR3 | DDR4 | DDR5 |
| --- | --- | --- | --- |
| Typical speed | 800–2133 MT/s | 1600–3200 MT/s | 4800–8400+ MT/s |
| Typical voltage | 1.5 V (1.35 V for DDR3L) | 1.2 V | 1.1 V |
| Key improvement | Older, lower-cost memory standard | Better performance and efficiency than DDR3 | Much higher bandwidth and improved efficiency |
| Typical use cases | Older PCs/laptops, legacy systems | Many PCs/laptops from the mid-2010s to early 2020s | Newer desktops, laptops, workstations and gaming PCs |
| Compatibility | DDR3 slots only | DDR4 slots only | DDR5 slots only |

Important: DDR3, DDR4 and DDR5 are not physically or electrically interchangeable. The motherboard must support the specific generation.

2.

Open your laptop or desktop (with power disconnected), locate the RAM slots, and take a clear photo of the installed RAM module. Reinstall the RAM module and verify that your system boots up successfully.<br><br><em><strong>Hint:</strong> Refer to your device manual or a trusted YouTube guide if unsure about RAM removal and installation steps.</em>

## RAM removal, photo, and reinstallation

I can't physically open your computer, take a photo, or verify its boot process. If you're doing this yourself, use this procedure:

- Shut the computer down completely and disconnect the power cable/charger.

- If it is a laptop, remove the battery if the manufacturer permits it.

- Ground yourself to reduce static-electricity risk.

- Open the device and locate the RAM slots.

- Push the retaining clips outward and carefully remove the RAM module by its edges. Do not touch the gold contacts.

- Take a clear photo showing the installed RAM module and its label.

- Align the notch on the RAM with the key in the slot.

- Press the module down firmly and evenly until the retaining clips click into place.

- Close the computer, reconnect power, and boot it.

- Confirm that the operating system recognizes the expected amount of RAM.

If you are unsure, use your manufacturer's service manual or a reputable video specifically for your computer model. Don't force the module—RAM installation varies somewhat between desktop DIMMs and laptop SO-DIMMs.

3.

List three key differences between HDD, SSD, and NVMe storage types, and for each, name one app or use case (from apps you use daily) that would benefit most from that storage type.

## HDD vs. SSD vs. NVMe

| Storage type | Three key differences | App/use case that benefits most |
| --- | --- | --- |
| HDD | Uses spinning magnetic platters; slowest of the three; inexpensive and offers high capacity | File storage/backup — useful for storing large collections of photos, videos, and documents where speed isn't critical |
| SSD (SATA) | Uses flash memory; much faster and quieter than HDD; usually connects through SATA | Everyday apps/browser — faster Windows/macOS startup and quicker launching of commonly used applications |
| NVMe SSD | Uses flash memory; connects through PCIe/NVMe rather than SATA; generally provides the highest throughput and lowest latency | Video editing or large games — benefits from very fast loading and large-file transfers |

A useful distinction is that NVMe is a storage interface/protocol, while SATA SSDs also use flash memory. An NVMe drive is therefore an SSD, but not every SSD is NVMe.

4.

Remove the storage drive (HDD or SSD) from your system, take a photo of the connector type (SATA, M.2, or NVMe), and reinstall the drive. Confirm that your device recognizes the storage after reassembly.<br><br><em><strong>Constraint:</strong> Only perform this task if you have access to a device you are allowed to open. If not, find and share a YouTube video link showing the process for your laptop/PC model.</em>

## Storage-drive removal and connector photo

Only do this if you have permission to open the computer and the manufacturer's instructions allow it.

For the practical portion, you can:

- Shut down and disconnect the computer from power.

- Open the system according to the manufacturer's service instructions.

- Locate the storage drive.

- Identify the connector:

  - SATA: separate SATA data and power connections, commonly used by 2.5-inch SSDs and HDDs.

  - M.2: a small, flat module inserted directly into an M.2 slot.

  - M.2 NVMe: an M.2 drive using the PCIe/NVMe interface.

- Take a clear photo of the connector/drive.

- Remove the drive carefully.

- Reinstall it securely.

- Reassemble the computer and boot it.

- Check File Explorer/Finder or the operating system's disk-management utility to confirm that the drive and its data are recognized.

If you don't have a device you're allowed to open, don't disassemble one just for this assignment. Give me your exact laptop/desktop model (for example, Dell Inspiron 15 3520), and I can find an appropriate YouTube demonstration for removing and reinstalling its storage drive.