1.

List all the main components you find inside a desktop PC after opening the cabinet, and write a short description (1-2 lines) for each.

# Desktop PC Disassembly and Reassembly Practical

## 1. Main Components Inside a Desktop PC

After opening a typical desktop PC cabinet, the following main components can be found:

- Motherboard — The main circuit board of the computer. It connects the CPU, RAM, storage devices, expansion cards, and other components.

- CPU (Processor) — The main processing unit of the computer. It executes instructions and performs calculations needed to run programs.

- CPU Cooler/Heat Sink — Removes heat produced by the processor. It usually consists of a metal heat sink and a fan.

- RAM (Memory) — Temporary, high-speed memory used by the CPU to store data and instructions that are currently being used.

- HDD/SSD (Storage Drive) — Stores the operating system, applications, and personal files. HDDs use magnetic disks, while SSDs use flash memory and have no moving parts.

- Power Supply Unit (PSU) — Converts AC electricity from the wall outlet into the DC voltages required by the PC components.

- Graphics Card (GPU) — Processes graphics and sends the resulting video signal to the monitor. Some computers use graphics integrated into the CPU instead.

- Case/Cabinet — The enclosure that holds and protects the computer's internal components.

- Cooling Fans — Move air through the cabinet to keep components such as the CPU, GPU, and motherboard at safe temperatures.

- Optical Drive — A CD/DVD drive found in some older or specialized desktop PCs. It reads and sometimes writes optical discs.

- CMOS Battery — A small coin-cell battery on the motherboard that helps maintain BIOS/UEFI settings and the system clock when the computer is powered off.

- Expansion Cards — Additional circuit boards installed in PCIe slots, such as sound cards, network cards, or capture cards.

- Data and Power Cables — Cables connect storage devices and other components to the motherboard and power supply.

# 2. Desktop PC Disassembly Procedure

Documentation note: Since an actual PC and camera are not available here, the diagrams below can be used as labeled diagrams in place of photographs. If this is a practical assignment, replace or supplement them with your own photographs of each step.

## Step 1 — Shut Down and Unplug the PC

Shut down the operating system completely. Switch off the power supply, unplug the power cable, and disconnect the monitor, keyboard, mouse, and other external devices.

Caption: The computer was completely powered off and disconnected from mains electricity before opening the cabinet.

WALL SOCKET

│

│  Power cable

▼

┌───────────┐

│    PSU    │

│  [ OFF ]  │

└───────────┘

│

PC CASE

Why: This reduces the risk of electric shock, short circuits, and damage to components.

## Step 2 — Remove the Side Panel

Place the PC on a stable table. Remove the screws or thumbscrews securing the side panel and slide the panel away from the case.

Caption: The side panel was removed to gain access to the internal components.

┌─────────────────────────┐

│       PC CABINET        │

│                         │

│  ┌───────────────────┐  │

│  │   SIDE PANEL      │  │ ← Remove

│  └───────────────────┘  │

└─────────────────────────┘

## Step 3 — Observe and Document the Components

Before disconnecting anything, inspect the internal layout and take photographs or make notes of cable positions.

Caption: The internal components and cable connections were documented before disassembly so that reassembly would be easier.

┌────────────────────────────────┐

│            CASE                │

│                                │

│  ┌──────────┐     ┌─────────┐ │

│  │   PSU    │     │  HDD/   │ │

│  │          │     │  SSD    │ │

│  └──────────┘     └─────────┘ │

│                                │

│  ┌──────────────────────────┐  │

│  │       MOTHERBOARD        │  │

│  │ CPU + Cooler             │  │

│  │ RAM   PCIe/GPU           │  │

│  └──────────────────────────┘  │

└────────────────────────────────┘

## Step 4 — Disconnect Internal Cables

Carefully disconnect the motherboard power connector, CPU power connector, storage data cables, storage power cables, GPU power cables, and front-panel/fan connectors as applicable.

Caption: Internal cables were disconnected carefully, and their locations were recorded for correct reassembly.

PSU ───────► Motherboard

PSU ───────► HDD/SSD

Motherboard ───► HDD/SSD

Motherboard ───► Front Panel

Motherboard ───► Fans

Tip: Pull connectors by their plugs rather than pulling on the wires.

## Step 5 — Remove the RAM

Open the locking clips at the ends of the RAM slot. Hold the RAM module by its edges and lift it straight out.

Caption: The RAM module was released from its slot and removed by holding its edges.

Before                    After

┌─────────────┐           ┌─────────────┐

│    RAM      │           │             │

│   MODULE    │           │   EMPTY     │

└─────────────┘           │    SLOT     │

▲       ▲               └─────────────┘

locking clips

Important: Avoid touching the gold contacts.

## Step 6 — Remove the HDD/SSD

Disconnect the drive's power and data cables. Remove the screws or release the drive mounting mechanism, then carefully remove the drive.

Caption: The storage drive was disconnected and removed from its mounting bay.

┌──────────────────────┐

│      HDD / SSD       │

│                      │

└──────────────────────┘

▲            ▲

Data cable   Power cable

## Step 7 — Remove the Graphics Card

If a dedicated graphics card is installed, disconnect its power cable if present. Remove the screw securing it to the case and release the PCIe slot latch. Carefully pull the card straight out.

Caption: The graphics card was released from the PCIe slot and removed while supporting it by its edges.

## Step 8 — Remove the CPU Cooler and CPU

Disconnect the CPU fan cable. Release the cooler according to its mounting mechanism and remove it carefully. If the CPU itself must be removed, release the CPU socket retention mechanism and lift the processor by its edges.

Caption: The CPU cooler was removed carefully, followed by the processor when required.

Note: For a basic disassembly exercise, it is safer to leave the CPU installed in its socket unless removal is specifically required.

## Step 9 — Remove the Motherboard

Disconnect any remaining cables. Remove the screws holding the motherboard to the case's standoffs. Support the motherboard with both hands and carefully lift it out.

Caption: The motherboard mounting screws were removed, and the motherboard was lifted out while avoiding contact with delicate components.

┌─────────────────────────────┐

│        MOTHERBOARD           │

│                             │

│ CPU     RAM       PCIe      │

│ [●]    [||||]     [====]    │

│                             │

└─────────────────────────────┘

▲    ▲    ▲    ▲

│    │    │    │

Screws/Standoffs

# 3. Desktop PC Reassembly Procedure

Reassembly should generally be performed in the reverse order of disassembly, while making sure that each component is properly seated and connected.

## Step 1 — Install the Motherboard

Place the motherboard onto the correct case standoffs and align the rear ports and mounting holes. Install the screws without overtightening them.

Caption: The motherboard was positioned on the case standoffs and secured with the correct screws.

Why the order matters: The motherboard provides the connection points for many other components, so it should be installed before connecting most internal components.

## Step 2 — Install the CPU and CPU Cooler

If the CPU was removed, place it into the socket according to the alignment marker and secure the socket mechanism. Install the cooler and connect its fan cable to the CPU-fan header.

Caption: The CPU and cooler were installed and the cooler fan was connected to the motherboard.

Why: The CPU must have its cooler properly installed before the computer is operated to prevent overheating.

## Step 3 — Install the RAM

Align the notch in the RAM module with the key in the motherboard slot. Press evenly until the retaining clips lock the module into place.

Caption: The RAM was aligned with the slot and firmly seated until the retaining clips locked it.

## Step 4 — Install the HDD/SSD

Place the storage drive into its mounting position and secure it. Connect its data cable to the motherboard and its power cable to the PSU.

Caption: The storage drive was mounted securely and connected with both data and power cables.

## Step 5 — Install the Graphics Card

Insert the graphics card into the appropriate PCIe slot until it is fully seated. Secure it to the case and reconnect its power cable if required.

Caption: The graphics card was installed into the PCIe slot and secured to the case.

## Step 6 — Reconnect Power and Data Cables

Reconnect the motherboard's main power connector, CPU power connector, storage power/data cables, GPU power cable, fan connectors, and front-panel connectors.

Caption: All previously documented cables were reconnected to their correct locations.

Why: Correct cable connections are necessary for the components to receive power and communicate with one another.

## Step 7 — Check All Connections

Before closing the case, check that RAM, GPU, cables, storage devices, and motherboard connections are properly seated.

Caption: All components and connections were checked before powering on the computer.

## Step 8 — Replace the Side Panel

Carefully put the side panel back on and secure it with its screws.

Caption: The side panel was replaced after confirming that all internal components were correctly installed.

## Step 9 — Reconnect External Devices and Test

Reconnect the power cable, monitor, keyboard, mouse, and other required peripherals. Turn on the PSU and start the computer.

Caption: The external cables were reconnected and the PC was powered on to verify successful reassembly.

If the computer starts normally, check that the RAM, storage drive, USB ports, fans, and display are recognized.

# 4. Safety Precautions and Tools Checklist

## Safety Precautions

- Shut down the computer completely. Never disassemble a running PC.

- Unplug the power cable. This disconnects the computer from mains electricity.

- Switch the PSU off. Set the PSU switch to O before working inside the case.

- Disconnect all external devices. Remove monitor, USB, network, and other cables.

- Work on a clean, dry, stable surface. This prevents components from falling or being exposed to moisture.

- Use anti-static precautions. Wear an anti-static wrist strap or regularly touch an appropriate grounded metal part of the case while it is disconnected from power.

- Hold components by their edges. Avoid touching gold contacts, chips, and exposed circuitry.

- Do not force connectors or components. If something does not fit, check its alignment and locking mechanism.

- Keep screws organized. Store screws in labeled containers so the correct screws are used during reassembly.

- Take photographs before disconnecting cables. Photos provide a reference for correct reassembly.

- Do not open the PSU casing. Power supplies can contain components that retain dangerous electrical energy even after being unplugged.

- Keep liquids away from the work area. Liquid spills can cause electrical shorts and component damage.

- Do not power on the PC with the CPU cooler removed. The processor can heat up rapidly without adequate cooling.

## Tools Needed

- Phillips-head screwdriver — Used for most motherboard, drive, expansion-card, and case screws.

- Small container or magnetic screw tray — Keeps screws organized during disassembly.

- Anti-static wrist strap — Helps reduce the risk of electrostatic discharge damaging components.

- Flashlight — Helps identify connectors and screws in dark areas of the case.

- Cable labels or masking tape — Can be used to label unfamiliar cables before removal.

- Camera or smartphone — Used to photograph cable positions and document each practical step.

- Clean, dry work surface — Provides a safe place to put components after removal.

- Compressed air/dust blower, if available — Can be used to remove dust, provided it is used appropriately and does not cause fans to spin excessively.

## Final Checklist Before Switching On

- Motherboard is securely mounted on the correct standoffs.

- CPU and CPU cooler are correctly installed.

- CPU fan cable is connected.

- RAM is fully seated.

- HDD/SSD is securely mounted and connected.

- Graphics card is fully seated and secured, if installed.

- Motherboard power connector is connected.

- CPU power connector is connected.

- Storage power and data cables are connected.

- Front-panel and fan connectors are connected correctly.

- No loose screws or tools have been left inside the case.

- Cables are clear of fans.

- Side panel is securely fitted.

- Monitor and other required peripherals are connected.

## Conclusion

Desktop PC disassembly and reassembly should be performed slowly and systematically. The most important principles are to disconnect power first, protect components from static electricity, document cable positions, handle components by their edges, and check every connection before powering the computer back on. Following the correct order makes the process safer and reduces the chance of damaging hardware.